# Packet Tracer - Troubleshoot Connectivity Issues

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
| R1 | G0/1 | 172.16.2.1 | 255.255.255.0 | N/A |
| R1 | S0/0/0 | 209.165.200.226 | 255.255.255.252 | N/A |
| R2 | G0/0 | 209.165.201.1 | 255.255.255.224 | N/A |
| R2 | S0/0/0 (DCE) | 209.165.200.225 | 255.255.255.252 | N/A |
| PC-01 | NIC | 172.16.1.3 | 255.255.255.0 | 172.16.1.1 |
| PC-02 | NIC | 172.16.1.4 | 255.255.255.0 | 172.16.1.1 |
| PC-A | NIC | 172.16.2.3 | 255.255.255.0 | 172.16.2.1 |
| PC-B | NIC | 172.16.2.4 | 255.255.255.0 | 172.16.2.1 |
| Web | NIC | 209.165.201.2 | 255.255.255.224 | 209.165.201.1 |
| DNS1 | NIC | 209.165.201.3 | 255.255.255.224 | 209.165.201.1 |
| DNS2 | NIC | 209.165.201.4 | 255.255.255.224 | 209.165.201.1 |

# Objectives

In this Packet Tracer activity, you will troubleshoot and resolve connectivity issues, if possible. Otherwise, the issues should be clearly documented so they can be escalated.

# Background / Scenario

Users are reporting that they cannot access the web server, www.cisco.pka after a recent upgrade that included adding a second DNS server. You must determine the cause and attempt to resolve the issues for the users. Clearly document the issues and any solution(s). You do not have access to the devices in the cloud or the server www.cisco.pka. Escalate the problem if necessary.

Note: Router R1 can only be accessed using SSH with the username Admin01 and password cisco12345. Router R2 is in the ISP cloud and is not accessible by you.

# Instructions

## Determine connectivity issues from PC-01.

On PC-01, open the command prompt. Enter the command ipconfig to verify what IP address and default gateway have been assigned to PC-01. Correct as necessary according to the Addressing Table.

After verifying/correcting the IP addressing issues on PC-01, issue pings to the default gateway, web server, and other PCs. Were the pings successful? Record the results.

### Questions:

Ping to default gateway (172.16.1.1)?    Yes

Type you answers here.

To web server (209.165.201.2)?   Yes

Type you answers here.

Ping to PC-02?   Yes

Type you answers here.

To PC-A?   No

Type you answers here.

To PC-B?   No

Type you answers here.

Use the web browser to access the web server on PC-01. Access the web server by first entering the URL http://www.cisco.pka and then by using the IP address 209.165.201.2. Record the results.

### Questions:

Can PC-01 access www.cisco.pka?   Yes

Type you answers here.

Using the web server IP address?   Yes

Type you answers here.

Document the issues and provide the solution(s). Correct the issues if possible.   IP address of PC-01 was wrong when we configured, so we went to Desktop IP Configuration, and corrected it. Also. We could not ping PC-A and PC-B, but it is not PC-01’s fault, we have to sort that out when we work on PC-A and PC-B.

Type your answers here.

## Determine connectivity issues from PC-02.

On PC-02, open the command prompt. Enter the command ipconfig to verify the configuration for the IP address and default gateway. Correct as necessary.

After verifying/correcting the IP addressing issues on PC-02, issue pings to the default gateway, web server, and other PCs. Were the pings successful? Record the results.

### Questions:

Ping to default gateway (172.16.1.1)?    Yes

Type you answers here.

To web server (209.165.201.2)?   Yes

Type you answers here.

Ping to PC-01?   Yes

Type you answers here.

To PC-A?   No

Type you answers here.

To PC-B?   No

Type you answers here.

Navigate to www.cisco.pka using the web browser on PC-02. Record the results.

Questions:

Can PC-02 access www.cisco.pka?   Yes

Type you answers here.

Using the web server IP address?   Yes

Type you answers here.

Document the issues and provide the solution(s). Correct the issues if possible.

- Default gateway of PC-01 was wrong, so we corrected it by going to Desktop IP Configuration. Also, we could not ping PC-A and PC-B, but we can not sort that out from PC-02, we have to check PC-A and PC-B for the solution.

Type your answers here.

## Determine connectivity issues from PC-A.

On PC-A, open the command prompt. Enter the command ipconfig to verify the configuration for the IP address and default gateway. Correct as necessary.

After correcting the IP addressing issues on PC-A, issue the pings to the web server, default gateway, and other PCs. Were the pings successful? Record the results.

### Questions:

To web server (209.165.201.2)?   No

Type you answers here.

Ping to default gateway (172.16.2.1)?   No

Type you answers here.

Ping to PC-B?   Yes

Type you answers here.

To PC-01?   No

Type you answers here.

To PC-02?   No

Type you answers here.

Navigate to www.cisco.pka using the web browser on PC-A. Record the results.

### Questions:

Can PC-A access www.cisco.pka?   Host Name Unresolved

Type you answers here.

Using the web server IP address?   Request Timeout

Type you answers here.

Document the issues and provide the solution(s). Correct the issues if possible.   We could not ping web server, default gateway, PC-01, PC-02. Also, we could not access PC-A using the web browser. The issue was, in R1, g0/1’s IP address was assigned wrong, so we used SSH command on PC-01 to assign the right IP address.

Type your answers here.

## Determine connectivity issues from PC-B.

On PC-B, open the command prompt. Enter the command ipconfig to verify the configuration for the IP address and default gateway. Correct as necessary.

After correcting the IP addressing issues on PC-B, issue the pings to the web server, default gateway, and other PCs. Were the pings successful? Record the results.

### Questions:

To web server (209.165.201.2)?   Yes

Type you answers here.

Ping to default gateway (172.16.2.1)?   Yes

Type you answers here.

Ping to PC-A?   Yes

Type you answers here.

To PC-01?   Yes

Type you answers here.

To PC-02?   Yes

Type you answers here.

Navigate to www.cisco.pka using the web browser. Record the results.

### Questions:

Can PC-B access www.cisco.pka?   Host Name Unresolved

Type you answers here.

Using the web server IP address   Yes

Type you answers here.

Document the issues and provide the solution(s). Correct the issues if possible.  PC-B could not access www.cisco.pka as it used DNS2, but all the other PCs used DNS1.

e your answers here.

Could all the issues be resolved on PC-B and still make use of DNS2? If not, what would you need to do? No, as we have to give a particular IP address to the DNS server, but it is not possible to do by going to command prompt.

- One possible solution is, we can change it to DNS1’s IP address by going to Desktop IP configuration DNS server.

Type your answers here.

## Verify connectivity.

Verify that all the PCs can access the web server www.cisco.pka.

Your completion percentage should be 100%. If not, verify that the IP configuration information is correct on all devices and that it matches what is shown in the addressing table.

End of document